# Robotic Arm Control System

### Project Description

The primary goal of this project is to create a  robotic arm capable of manipulating small objects. The control system is built around an Arduino UNO, which manages four Servo Motors and a 2-axis Analog Joystick. Three servos are dedicated to the arm's movement, while the fourth controls the gripper mechanism. Initially, all servos are set to a neutral 90 position. The arm's movement is fully controlled by the operator: the joystick's X-axis controls the base rotation (left/right), the Y-axis controls the forward/backward reach, and the vertical movement (up/down) is controlled incrementally via serial input ('a' and 'd' keys). The gripper is actuated by pressing the joystick's integrated button.

Components

- Arduino UNO

- 4 SG90 Servo Motor

- Analog Joystick

- Breadboard & Jumpers

## Operational Flow

- Read Cycle: The Arduino enters a tight loop, reading the state of all inputs (Joystick, Button, Serial) every  15 milliseconds.

- Angle Calculation: The Control Module modifies the current servo angles incrementally based on the inputs.

- Gripper Toggle: Button press instantly toggles the gripper between its two defined states (Open/Semi-Closed).

- Movement Execution: The Actuator Module writes the new, constrained angles to the corresponding servos.

- Real-Time Response: The low-latency loop ensures the arm reacts smoothly and predictably to manual inputs.

## Hardware Connections

- Servo Motors (PWM): Connected to Digital Pins 6, 9, 10, 11 (dedicated PWM pins).

- Servo Power: Connected to an External 5V PSU. Critical: Must share a common GND with the Arduino.

- Joystick VRX: Connected to Analog A0 (Base rotation input).

- Joystick VRY: Connected to Analog A1 (Arm reach input).

- Button: Connected to Digital Pin 2. Configured as INPUT_PULLUP, connected to GND when pressed.